AP010 Project 2

Engineer a multi-track recording of a two to three minute musical performance. Utilize at least three different acoustic musical instruments.  One of which must be a minimum of four mics on acoustic drums.  You may record additional DI instruments such as electronic keys or bass – No MIDI/Loops.  Overdubs are encouraged.  Don’t overdo it because you will need to mix this project on the Toft for your final project.

Properly name the session.  For example:

Your_Name_Project2

Use 48kHz/24bit session parameters.

Utilize at least two different stereo microphone techniques such as XY, spaced pair, or MS, and at least two different placement techniques such as close, distant, or ambient.  Correct use of MS earns 2 extra credit points.

Try to set input levels that result in unity fader levels at mix.

Do not employ any mix processing except proper panning.

Session Organization:  Properly label tracks what they are.  For example guitar, vocal, snare.

Use the comments section to detail mic types and placement.

Utilize memory location markers to identify important locations of the recording such as the top, verse, chorus, bridge, and end.

Burn to CD or DVD-R as mac/pc data properly labeled with a Sharpie marker –

For example:

Your_Name_Project2

AP010

08.23.10

Instructor: J_Taylor

## Project 2 Grade Rubric

- Content:

16 points – Project contain at least three live acoustic instruments.

- Drums:

16 points – Project must include a minimum of four tracks devoted to live acoustic drums recorded with good separation on close mics and good coverage on distance mics.

- 2 Stereo Techniques:

16 points – Project must include at least two stereo mic techniques.  Tracks must be properly panned and/or decoded.

- 2 Placement Techniques:

16 points – Project must include at least two placement techniques.

- Recording Quality:

20 points – All tracks must exhibit acceptable gain, distortion, and signal to noise characteristics… good levels with no hiss and no distortion.

- Organization:

10 points - Disc and session must be properly labeled.  Tracks should be labeled by source name.  Comments should include mic and placement information.

- Musicality:

5 points – Session must exhibit some degree of musicality.  Composition can be original or a cover.  No random bleeps, blurps, or blats.  You want to be able to use this on your portfolio.

Total: 100 project points = 25 Course Points

Sessions with missing files will receive a zero!

Late projects will not be accepted.